# Runbook – Network Alerts

## Purpose

Provide a clear, repeatable procedure to detect, triage, correlate, mitigate, and document network-specific alerts for production environments across Arista, Palo Alto Networks, Cisco, OpenGear, and Aruba devices.

## Scope

In scope:

- Production network devices from Arista, Palo Alto, Cisco, OpenGear, and Aruba.

- Includes routers, switches, firewalls, and out-of-band management devices.

- Covers physical appliances.

Out of scope:

- Cloud-managed networking services (e.g., Meraki, Aruba Central) – escalate to cloud provider.

- Non-production environments unless explicitly included in monitoring policy.

- Assumes:

  - Metrics platform (SNMP, telemetry, or streaming analytics).

  - Centralized log platform (e.g., syslog, SIEM).

  - Incident/ticketing system.

## Triggers

Alerts fire when critical network health indicators breach thresholds for sustained periods (≥5 minutes).

### Arista

- Device unreachable or SNMP polling failure.

- BGP session down or flapping.

- High CPU (>85%) or memory (>80%) utilization.

- Interface errors (CRC, drops) or high utilization (>80%).

- MLAG peer down or inconsistent state.

### Palo Alto Networks

- Firewall unreachable or HA pair state degraded.

- High session count (>80% of capacity).

- Zone protection or DoS alerts triggered.

- Critical system logs (e.g., dataplane restart).

- High CPU or memory usage on dataplane/control plane.

### Cisco

- Device unreachable or SNMP timeout.

- Interface down or err-disabled.

- High CPU (>85%) or memory (>80%).

- Routing protocol adjacency loss (OSPF, EIGRP, BGP).

- Hardware errors (e.g., line card failure).

### OpenGear

- OOB device unreachable.

- Cellular link down or degraded.

- High CPU or memory usage.

- Serial port connectivity failure.

### Aruba

- Controller unreachable or cluster degraded.

- AP down or disconnected.

- High client count exceeding thresholds.

- Authentication failures spike.

- High CPU/memory on controller.

## Gather Context

- Device type, model, and OS version.

- Host identity: hostname, IP, environment, region/site.

- Role and criticality: core, fabric, distribution, access, firewall.

- Change context: recent config changes, firmware upgrades.

- Network topology: upstream/downstream dependencies.

## Initial Validation

- Confirm alert timestamp, duration, and persistence.

- Determine blast radius: single device vs multiple devices.

- Check for maintenance windows or planned changes.

- Validate monitoring source accuracy (SNMP, syslog, telemetry).

## Rapid Triage – Finding a Pattern

- Connectivity issues: Ping/traceroute to device and upstream peers.

- Interface errors: Review counters for CRC, drops, discards.

- Routing instability: Check protocol logs and neighbor states.

- Firewall session spikes: Correlate with traffic patterns or attack signatures.

- Cluster health: Validate HA/MLAG/VRRP states.

## Data Collection (High Level)

- Current alert metrics (CPU, memory, interface stats).

- Device logs around alert window.

- Routing table and neighbor status.

- HA/cluster status.

- Recent configuration changes.

## Correlation Signals

- Align metrics, logs, and change events by timestamp.

- Identify recurring patterns:

  - Firmware upgrade overlap.

  - Traffic surge or DDoS indicators.

  - Physical link degradation (fiber/cable issues).

## Suggest Remediation (Approvals Required)

- Interface errors: Reseat cables, replace optics, adjust speed/duplex.

- Routing instability: Clear sessions, restart protocol, escalate if persistent.

- Firewall overload: Drop non-critical sessions, apply rate-limiting.

- Cluster issues: Failover to healthy node, restart services.

- Device unreachable: Power cycle via OOB, escalate to onsite support.

## Validate Recovery

- Ensure alert clears and remains stable for 10–15 minutes.

- Verify network path health and application connectivity.

- Confirm routing and HA states are normal.

## Documentation Requirements

Record in incident/ticket:

- Device type, model, OS version, role, environment.

- Alert details: metric, threshold, start/stop.

- Evidence: metrics summary, logs, interface stats.

- Actions taken, approvals, and outcomes.

- Impact assessment and follow-up tasks.

## Threshold Tuning and Alert Hygiene

- Require persistence (≥5 minutes) to avoid transient spikes.

- Adjust thresholds based on device role and traffic patterns.

- Suppress alerts during maintenance windows.

- Include HA and routing health checks for critical devices.